# Comprehensive Pedagogical Assessment and Objective Evaluation Framework: Computer Organization and Architecture

## Theoretical Framework and Cognitive Assessment Paradigm

The structural intricacies of Computer Organization and Architecture require an evaluation mechanism that transcends rote memorization, demanding spatial visualization, algorithmic tracing, and profound micro-architectural synthesis. Based on the standard Bachelor of Computer Application (BCA) Semester 2 curriculum parameters, the syllabus is partitioned into five distinct cognitive domains, traversing fundamental Register Transfer Language (RTL) logic to advanced memory hierarchy stratification.1

An effective objective-based evaluation must map this cognitive progression algorithmically. The subsequent repository constructs an exhaustive, 200-question assessment framework distributed strictly alongside the mandated five-unit syllabus. To balance the cognitive load and ensure a rigorous statistical distribution of student proficiency, the questions are calibrated into three tiers: 40% foundational recall (Easy), 40% analytical application (Medium), and 20% complex computational synthesis and cognitive traps (Hard). Furthermore, the evaluation integrates targeted spatial and computational traps. These include effective address miscalculations in Program Counter (PC) relative modes, multi-variable Karnaugh Map (K-map) visualization errors, and cache mapping index discrepancies, which are critical for distinguishing surface-level familiarity from nuanced architectural mastery.3

## UNIT-I: Register Transfer Level, Micro-Operations, and Arithmetic Algorithms

The micro-architectural foundation dictates how data traverses the internal datapath. The pedagogical emphasis relies on visualizing Register Transfer Language (RTL) not as abstract code, but as physical hardware pathways controlled by multiplexers and tri-state buffers. A primary diagnostic indicator of architectural mastery is the application of Booth's Multiplication Algorithm for signed two's complement numbers. Visual learning paradigms dictate that students should mentally map the sequence counter and the  bit pairs as a sliding window across the multiplier.6 Cognitive traps frequently occur during the arithmetic right shift phase of this algorithm, where candidates fail to preserve the sign bit, leading to cascading errors in the accumulator.8 Assessment patterns heavily emphasize the transition logic: a multiplier bit pair of  dictates the subtraction of the multiplicand, whereas  dictates addition.6 Moreover, calculating the exact number of logical gates or multiplexers necessary for specific common bus architectures serves as a rigorous vector for medium- and hard-difficulty evaluation.

### Unit I Assessment Matrix

| # | Difficulty | Question | Options | Correct Answer | Detailed Pedagogical Explanation |
| --- | --- | --- | --- | --- | --- |
| 1 | Easy | Which notation is conventionally utilized to represent the hardware transfer of data from register R1 to R2? | A) R1 -> R2 B) R2 <- R1 C) R1 == R2 D) R2 = R1 | B | In standard RTL, a left-pointing arrow denotes the physical routing of data into the destination register from a source. |
| 2 | Easy | A common bus system servicing 4 registers of 8 bits each necessitates how many individual multiplexers? | A) 4 B) 8 C) 16 D) 32 | B | The architecture requires one multiplexer for each bit position across the registers, totaling 8 multiplexers. |
| 3 | Easy | What is the requisite dimension of each multiplexer required to construct a common bus for 8 registers of 16 bits each? | A) 4:1 B) 8:1 C) 16:1 D) 32:1 | B | The multiplexer size is governed strictly by the number of source registers (8), requiring 3 selection lines. |
| 4 | Easy | Which micro-operation shifts a binary operand to the left while deterministically inserting a logic 0 into the Least Significant Bit (LSB)? | A) Arithmetic right B) Logical left C) Circular right D) Arithmetic left | B | Logical shifts treat data as unsigned binary, systematically inserting 0s into all vacated bit positions. |
| 5 | Easy | During an arithmetic shift right operation, what transformation occurs to the Most Significant Bit (MSB)? | A) It is inverted B) It is shifted out C) It remains unchanged D) It is replaced by 0 | C | Arithmetic right shifts duplicate the MSB (the sign bit) to maintain the value's mathematical polarity. |
| 6 | Easy | Booth's multiplication algorithm is uniquely optimized to process which specific numeric representation natively? | A) Unsigned binary B) IEEE 754 Floating-point C) Signed 2's complement D) Binary Coded Decimal | C | Booth's algorithm inherently processes signed numbers without requiring pre-conversion to positive magnitudes.6 |
| 7 | Easy | What specific arithmetic manipulation is executed if the scrutinized Booth bits () are evaluated as 01? | A) Subtraction B) Addition C) Logical shift only D) Halting operation | B | The 01 sequence signifies the trailing edge of a string of 1s, necessitating the addition of the multiplicand.6 |
| 8 | Easy | What structural action is mandated in Booth's algorithm if the bit pair () evaluates to 11? | A) Add multiplicand B) Subtract multiplicand C) Arithmetic Right Shift only D) Reset Sequence Counter | C | Consecutive identical bits (11 or 00) require no arithmetic intervention, strictly a shift.6 |
| 9 | Easy | The core of the Arithmetic Logic Unit (ALU) is fundamentally constructed using which classification of digital circuits? | A) Sequential memory B) Combinational logic C) Analog comparators D) Delay lines | B | ALUs rely entirely on combinational logic gates for immediate, clock-independent mathematical processing. |
| 10 | Easy | Which architectural register is traditionally designated to hold temporary operands and resultant data during successive ALU operations? | A) Program Counter (PC) B) Accumulator (AC) C) Instruction Register (IR) D) Stack Pointer (SP) | B | The Accumulator acts as the primary data nexus for interim arithmetic and logical results. |
| 11 | Easy | The memory transfer operation that reads a designated word from the location specified by the Address Register (AR) is formally denoted as: | A) M <- R1 B) R1 <- M C) R1 <- AR D) AR <- M | B | Register R1 receives the fetched data payload from the specific memory index pointed to by AR. |
| 12 | Easy | Which specific shift micro-operation establishes a closed loop, routing the shifted-out MSB directly back into the vacant LSB? | A) Logical shift B) Arithmetic shift C) Circular shift (Rotate) D) Linear shift | C | Circular shifts (rotations) preserve all bits by feeding exiting bits back into the opposite end of the register. |
| 13 | Easy | Hardware subtraction of two operands is most efficiently realized in contemporary ALUs utilizing: | A) 1's complement addition B) 2's complement addition C) 9's complement logic D) Sign-magnitude comparators | B | 2's complement representation permits subtraction using identical parallel adder circuitry simply by asserting the carry-in bit. |
| 14 | Easy | Which designated flag within the ALU's status register asserts to indicate a negative mathematical result? | A) Zero (Z) B) Carry (C) C) Overflow (V) D) Sign (S) | D | The Sign flag is a direct physical reflection of the MSB of the ALU's output bus. |
| 15 | Easy | Tri-state buffers are critically deployed in shared bus architectures to furnish which specific operational state? | A) High impedance (Hi-Z) B) Signal amplification C) Memory persistence D) Error parity correction | A | The High Impedance state electrically isolates inactive register outputs from corrupting the shared data bus. |
| 16 | Easy | What is the precise 2's complement representation of the 4-bit binary operand 0101? | A) 1010 B) 1011 C) 1100 D) 1101 | B | The 1's complement is 1010; logically adding 1 to the LSB yields 1011. |
| 17 | Medium | A complex CPU features 32 general-purpose registers, each 64 bits wide. How many distinct selection lines are mandated for the bus multiplexers? | A) 3 B) 4 C) 5 D) 6 | C | The required selection lines are defined logarithmically: lines to uniquely address 32 sources. |
| 18 | Medium | When initializing Booth's algorithm to multiply -5 and -7 within a 4-bit architectural system, what is the initial multiplier (Q) encoded in 2's complement? | A) 1011 B) 0101 C) 1001 D) 1111 | C | The positive magnitude +7 is 0111; its 2's complement representation for -7 is 1001.6 |
| 19 | Medium | During the transitional state in Booth's algorithm, the RTL operation is mandated. How is this physically synthesized in the ALU hardware? | A) AC + BR B) AC + BR' C) AC + BR' + 1 D) AC - BR' | C | Subtraction is structurally executed by passing the multiplicand through an inverter (BR') and asserting the of the adder (+1).7 |
| 20 | Medium | In an arithmetic left shift micro-operation, what specific logic condition triggers the hardware Overflow (V) flag? | A) The shift-in bit evaluates to 1 B) The MSB differs from the adjacent bit before shifting C) The LSB state transitions D) The sequence counter reaches zero | B | Overflow indicates a sign reversal; it occurs if the MSB changes state during the leftward progression. |
| 21 | Medium | What is the fundamental architectural divergence between a tri-state buffer bus and a multiplexer-based shared bus? | A) Operating frequencies B) Decoding versus encoding logic C) High-impedance switching versus combinational logic routing D) Serial versus parallel transmission | C | Standard buses utilize tri-state high-impedance to prevent collisions; MUX buses use strict AND/OR combinational routing. |
| 22 | Medium | If an 4-bit accumulator contains 1011, what is the exact binary consequence of a single arithmetic right shift? | A) 0101 B) 1101 C) 1100 D) 0110 | B | The arithmetic right shift duplicates the MSB (1), shifting remaining bits right, discarding the LSB: 1101. |
| 23 | Medium | A hardware realization of Booth's algorithm employs a Sequence Counter (SC). What is the terminal initialization value for an 8-bit multiplier? | A) 7 B) 8 C) 9 D) 0 | B | The Sequence Counter is initialized to exactly , representing the bit-width of the multiplier.6 |
| 24 | Medium | For the memory write operation , which specific CPU control signal must be actively asserted? | A) Memory Read B) Memory Write C) Instruction Fetch D) Operand Decode | B | The Memory Write control strobe enables the RAM module to latch the data residing on the system data bus. |
| 25 | Medium | Which of the following RTL equations correctly formalizes a micro-operation subjected to a binary control condition ? | A) B) C) D) | A | The colon syntax definitively specifies the Boolean condition under which the subsequent transfer is authorized. |
| 26 | Medium | Multiplying (multiplicand) and (multiplier) using Booth's algorithm yields what signed decimal equivalent? | A) 12 B) -12 C) 24 D) -24 | A | resolves to -3 (5-bit 2's comp), and resolves to -4 (4-bit). The product .10 |
| 27 | Medium | What architectural mechanism explicitly prevents destructive bus contention in a shared common bus system? | A) The Instruction Register B) The System Clock C) Mutually exclusive Decoder output lines D) The Stack Pointer | C | A decoder guarantees that only one specific source register's tri-state buffers are enabled at any given microsecond. |
| 28 | Medium | If and , what is the result of the hardware micro-operation ? | A) 0110 B) 1000 C) 0100 D) 1110 | A | The Exclusive-OR (XOR) operation evaluates to 1 exclusively when corresponding input bits differ: . |
| 29 | Medium | Exactly how many processor clock cycles does Booth's algorithm consume to evaluate an -bit multiplier? | A) B) C) D) | C | The algorithm structurally evaluates every adjacent bit pair once, iterating identically times before termination. |
| 30 | Medium | To clear the contents of a register without routing zeros from the bus, which logical micro-operation is optimally applied to the register against itself? | A) AND B) OR C) XOR D) NAND | C | XORing any binary sequence with itself () inherently forces all bits to logical zero. |
| 31 | Medium | To selectively set specific bits within a target register to 1 without mutating the remaining bits, one must apply: | A) A selective-set (OR) mask B) A selective-clear (AND) mask C) A selective-complement (XOR) mask D) A logical right shift | A | Executing a logical OR with a mask containing 1s sets the target bits, while 0s leave the original bits unaltered. |
| 32 | Medium | In division algorithms, why is the restoring method characterized as structurally slower than the non-restoring method? | A) It requires dual multiplication sequences B) It necessitates an extra addition cycle to restore the partial remainder if subtraction yields a negative C) It mandates double-precision shifting D) It utilizes floating-point coprocessors | B | Restoring division wastes a clock cycle adding the divisor back to the accumulator whenever a trial subtraction fails. |
| 33 | Hard | Multiply the signed integers -9 and -13 using Booth's algorithm. What is the precise 8-bit binary product generated in the combined accumulator/multiplier registers? | A) 01110101 B) 00001111 C) 10001011 D) 01110011 | A | The decimal product is 117. Translated into 8-bit signed binary, 117 is mathematically represented as 01110101.8 |
| 34 | Hard | In a 64-bit microarchitecture, a bus system engineered using tri-state buffers to service 16 distinct registers requires exactly how many buffers in total? | A) 64 B) 1024 C) 256 D) 512 | B | The architecture demands one buffer per bit per register: tri-state buffers. |
| 35 | Hard | In Booth's algorithm, what specific sequence of Booth bit transitions () optimally handles a continuous block of 1s (e.g., the pattern 011110) in the multiplier? | A) Subtraction followed by immediate addition B) Initial subtraction, multiple null shifts, culminating in addition C) Continuous iterative addition D) Alternating add and subtract cycles | B | The algorithm detects the edge via a 10 (subtract), processes the block interior via 11 (shifts), and detects the end via 01 (add).6 |
| 36 | Hard | If a processor deploys a 32-bit ALU constructed via a simple ripple carry adder, what is the critical path delay trap that limits processor frequency? | A) Delay is constant time B) Delay is logarithmically proportional to C) Delay is linearly proportional to D) Delay is independent of the input operands | C | The carry bit must physically ripple through successive full adder stages, creating an propagation bottleneck.11 |
| 37 | Hard | A shift micro-operation is applied to an accumulator containing . Following exactly two circular left shifts, what is the resultant binary value? | A) 11001010 B) 01100101 C) 11001001 D) 11010010 | A | First shift: . Second shift loops the MSB (0) to the end: . |
| 38 | Hard | The Boolean control expression permits the data transfer to execute strictly under which logic state? | A) Both X and Y are 1 B) X is 0 and Y is 1 C) X is 1 and Y is 0 D) Both X and Y are 0 | B | The control function evaluates to a logical 1 (true) exclusively when variable X is 0 and variable Y is 1. |
| 39 | Hard | Within an ALU, implementing the subtraction operation requires the arithmetic circuit to set the Carry-in () to what specific state while adding the 1's complement of B? | A) 1 B) 0 C) Oscillating D) High Impedance | B | Standard subtraction is . By deliberately setting , the circuit yields , mathematically equivalent to . |
| 40 | Hard | Why is Booth's algorithm computationally inferior to standard shift-and-add algorithms when confronted with alternating bit patterns (e.g., 010101)? | A) It cannot process unsigned data B) It skips the shift step entirely C) It predicts the bits incorrectly D) It mandates an arithmetic operation on every single clock cycle | D | A trap question: Alternating bits cause constant 10 and 01 transitions, forcing worst-case performance by requiring add/subtracts every cycle instead of null shifts. |

## UNIT-II: Boolean Algebra, Logic Gates, and K-Map Simplification

Unit II transitions from physical datapath logic to the mathematical formulation of digital circuits. The pedagogical emphasis lies in Boolean algebraic theorems, canonical forms (Sum of Products and Product of Sums), and the spatial minimization of complex expressions via Karnaugh Maps. A critical differentiator for advanced architectural proficiency is the ability to navigate 5-variable and 6-variable K-maps. The spatial cognitive mapping of a 5-variable Karnaugh map necessitates visualizing two overlaid 4-variable sub-maps (yielding 32 total cells), where the planes differ exclusively by the Most Significant Bit (MSB). Similarly, 6-variable maps necessitate visualizing a hypercube of four 4-variable grids (64 cells).3

Evaluative traps frequently involve the identification of redundant groupings versus Essential Prime Implicants. Candidates often struggle with the duality of logic: extracting Sum of Products (SOP) by grouping logical 1s, versus formulating Product of Sums (POS) by grouping 0s and subsequently applying DeMorgan's involution.14 Implementation using universal logic (NAND/NOR cascades) tests the synthesis capabilities of translating abstract equations into viable silicon structures.

### Unit II Assessment Matrix

| # | Difficulty | Question | Options | Correct Answer | Detailed Pedagogical Explanation |
| --- | --- | --- | --- | --- | --- |
| 41 | Easy | Which foundational Boolean theorem states mathematically that ? | A) Consensus Theorem B) DeMorgan's Law C) Associative Law D) Distributive Law | B | DeMorgan's law dictates the inversion of individual variables coupled with the flipping of logical operators (OR to AND). |
| 42 | Easy | Which discrete logic gate is universally recognized for its ability to synthesize any other Boolean function independently? | A) AND B) XOR C) NAND D) NOT | C | NAND (alongside NOR) is a Universal Gate, capable of implementing AND, OR, and NOT operations through specific structural configurations. |
| 43 | Easy | A canonical Sum of Products (SOP) expression is structurally constructed entirely from which elements? | A) Maxterms B) Minterms C) Don't care conditions D) Unprimed literals | B | Canonical SOP is strictly defined as the logical sum (OR) of all applicable minterms (products containing all variables). |
| 44 | Easy | Precisely how many individual plotting cells comprise a standard 4-variable Karnaugh Map? | A) 8 B) 12 C) 16 D) 32 | C | A Karnaugh map for variables mathematically dictates cells; thus distinct cells.16 |
| 45 | Easy | In K-map simplification, successfully grouping a "quad" (four adjacent 1s) mathematically eliminates how many variables from the term? | A) 1 B) 2 C) 3 D) 4 | B | A pair eliminates 1 variable, a quad eliminates 2 variables, and an octet eliminates 3 variables, leaving only the constants. |
| 46 | Easy | Which Boolean algebraic law is physically represented by the absorption equation ? | A) Commutative Law B) Absorption Law C) Idempotent Law D) Consensus Theorem | B | The Absorption law simplifies logic by recognizing that the presence of renders the state of irrelevant. |
| 47 | Easy | The Product of Sums (POS) simplification technique focuses on grouping which specific values plotted on a K-map? | A) Logical 1s B) Logical 0s C) Don't Cares strictly D) Essential Prime Implicants | B | POS focuses on grouping maxterms, which correspond to the logical 0s present in the function's truth table.15 |
| 48 | Easy | Which logic gate asserts a logical 1 output exclusively when all of its inputs are identically matched? | A) XOR B) XNOR C) NAND D) OR | B | The Exclusive-NOR (XNOR) gate functions as a strict logical equivalence detector. |
| 49 | Easy | The Boolean expression unconditionally evaluates to what logical state? | A) 0 B) 1 C) A D) A' | B | Under the Law of Complementarity, a variable logically ORed with its own inverse must always yield a true (1) state. |
| 50 | Easy | Which mathematical format utilizes the notation for function declaration? | A) Canonical POS B) Canonical SOP C) Truth Table matrix D) Gray code sequence | B | The notation denotes the Sum of Minterms, which is the canonical SOP format. |
| 51 | Easy | To structurally implement a NOT gate utilizing a single NAND gate, what physical alteration must be applied to the inputs? | A) Invert one externally B) Ground one input C) Tie both inputs together D) Connect one to Vcc | C | Tying both NAND inputs together forces the gate to evaluate , synthesizing an inverter. |
| 52 | Easy | What is the precise logical dual of the Boolean expression ? | A) B) C) D) | A | Duality requires swapping AND/OR operators and simultaneously swapping 1/0 constants, while preserving variables. |
| 53 | Easy | Adjacent cells in a properly constructed K-map must logically differ by exactly how many bit positions? | A) 0 B) 1 C) 2 D) Variable | B | K-maps employ Gray code numbering along axes to ensure exactly a 1-bit unit distance between adjacent cells.15 |
| 54 | Easy | Which Boolean algebraic property structurally permits the transposition ? | A) Distributive Property B) Associative Property C) Commutative Property D) Absorptive Property | C | The Commutative property dictates that the sequence of operands does not alter the logical outcome. |
| 55 | Easy | A "Don't Care" (X) condition plotted on a K-map is conceptually treated as: | A) Strictly a logical 0 B) Strictly a logical 1 C) Either 0 or 1 to optimize grouping D) An invalid hardware state | C | Don't Cares act as wildcards, utilized as 1s or 0s depending on which choice yields the largest and simplest K-map loop. |
| 56 | Easy | How many distinct variables must a minterm contain within a 3-variable logic function? | A) 1 B) 2 C) 3 D) It varies | C | By definition, a standard minterm must incorporate all variables of the given function, whether primed or unprimed. |
| 57 | Medium | Simplify the Boolean expression leveraging the Consensus Theorem. | A) B) C) D) | A | The Consensus theorem identifies as redundant because any condition making true also makes or true. |
| 58 | Medium | To accurately map a 5-variable logic function, how many distinct cells are required in the K-map architecture? | A) 16 B) 32 C) 64 D) 128 | B | The cell count is , typically represented visually as two adjacent 16-cell sub-maps.3 |
| 59 | Medium | When overlaying two 4-variable sub-maps to construct a 5-variable K-map, cells occupying the same relative coordinate on both maps differ by which specific variable? | A) The Least Significant Bit (LSB) B) The middle bit C) The Most Significant Bit (MSB) D) No bits differ | C | The MSB structurally separates the map into two distinct spatial planes: the plane and the plane.3 |
| 60 | Medium | Determine the minimum quantity of 2-input NAND gates required to synthesize an equivalent OR gate. | A) 2 B) 3 C) 4 D) 5 | B | Inverting both input signals requires 2 NANDs; feeding those inverted signals into a 3rd NAND gate completes the OR logic. |
| 61 | Medium | Simplify the function utilizing a K-map. | A) B) C) D) | A | Plotting the minterms reveals they occupy all four corners/edges where variable C is 0, simplifying cleanly to . |
| 62 | Medium | A grouping of 8 adjacent 1s (an octet) within a 4-variable K-map logically eliminates exactly how many variables? | A) 1 B) 2 C) 3 D) 4 | C | An octet simplifies the logical term down to a single remaining variable, mathematically eliminating 3 variables. |
| 63 | Medium | Derive the POS form for a truth table where the output is strictly 0 for input combinations 000 and 011. | A) B) C) D) | A | The maxterms and translate directly into the sum products and . |
| 64 | Medium | What terminology defines an implicant that covers at least one specific minterm not covered by any other prime implicant on the K-map? | A) Redundant Implicant B) Essential Prime Implicant C) Prime Implicant D) Ghost Implicant | B | Essential Prime Implicants are mathematically mandatory and must be included in the final minimized SOP expression.17 |
| 65 | Medium | Apply DeMorgan's theorem to simplify the negated product . | A) B) C) D) | A | The AND operators flip to OR operators, and each individual variable's logical state is inverted. |
| 66 | Medium | A 3-variable combinational circuit outputs 1 exclusively for inputs 1, 3, 5, and 7. What is the simplified Boolean function? | A) A B) B C) C D) A' | C | Examining the binary representation (001, 011, 101, 111), the output is highly correlated entirely to the state of . |
| 67 | Medium | Implement the Boolean function utilizing only NAND gates. What is the minimum gate count required? | A) 3 B) 4 C) 5 D) 6 | B | Standard optimal synthesis of an XOR gate utilizes exactly 4 interconnected NAND gates. |
| 68 | Medium | In a K-map, edge wrap-around grouping (connecting left edge to right edge) is geometrically valid due to which underlying mathematical principle? | A) Binary fractional addition B) Gray code cyclic adjacency C) DeMorgan's loopback D) The Consensus theorem | B | The edges are unit-distance apart; Gray code cyclic properties ensure the opposing edges differ by only one variable.15 |
| 69 | Medium | Calculate the minimum SOP for where represents don't care conditions. | A) C B) A'C C) B'C D) C + A' | A | The quad of 1s simplifies perfectly to . Incorporating the don't cares does not yield a larger group, so they are ignored. |
| 70 | Medium | To natively realize a Product of Sums (POS) expression, which universal gate cascaded structure is geometrically preferred? | A) NAND-NAND B) NOR-NOR C) AND-OR D) OR-AND | B | NOR-NOR logic structures natively and intuitively map to Product of Sums architectural implementations. |
| 71 | Medium | If two prime implicants are mapped such that they overlap entirely within the bounds of a larger implicant, they are classified as: | A) Essential B) Redundant C) Maxterms D) Invalid | B | Redundant groups must be eliminated from the final expression to achieve true mathematical minimization. |
| 72 | Medium | Which encoding sequence ensures that K-map column numbering proceeds as 00, 01, 11, 10? | A) Binary Coded Decimal B) ASCII Encoding C) Excess-3 Code D) Gray Code | D | Gray code deliberately rearranges binary progression to guarantee a unit-distance transition between any adjacent cells.15 |
| 73 | Hard | Simplify the 4-variable function . | A) B) C) D) | C | Visualizing the map allows grouping an octet covering the entire top half () and a quad in the bottom right ().18 |
| 74 | Hard | In a densely packed 6-variable K-map, exactly how many adjacent cells geometrically surround any single specific target cell? | A) 4 B) 5 C) 6 D) 8 | C | A fundamental rule of K-map topology is that in an -variable space, every cell shares an edge with exactly other cells. |
| 75 | Hard | How many 4-variable sub-maps must be logically tiled to constitute a complete 6-variable K-map? | A) 2 B) 4 C) 6 D) 8 | B | The 64 required cells () are divided into planes. A 4-variable map has 16 cells, therefore sub-maps are required.12 |
| 76 | Hard | Simplify within a 5-variable K-map structure. | A) B) C) D) | D | This complex octet bridges identical corner positions symmetrically across both the and logical planes. |
| 77 | Hard | Given the equation , implement it utilizing a single specific gate type. Which gate fulfills this? | A) 4-input NAND B) 4-input NOR C) 2-input XNOR D) 4-input AND | B | Applying DeMorgan's expansion yields , which is the exact definition of a 4-input NOR gate. |
| 78 | Hard | Determine the precise number of prime implicants for the function . | A) 2 B) 3 C) 4 D) 5 | B | Analytical grouping yields (0,2,4,6), (4,5), and (2,10), identifying exactly 3 prime implicants.11 |
| 79 | Hard | During POS minimization, plotting 0s yields an intermediate expression . What is the final function ? | A) B) C) D) | B | To finalize POS, DeMorgan's involution is applied to : , which evaluates to . |
| 80 | Hard | A custom logic circuit specifies active minterms . What standard macro-function does this circuit synthesize? | A) 3-input NAND B) 3-input XNOR C) 3-input XOR D) Majority voting gate | C | Minterms possessing an odd parity of 1s (001, 010, 100, 111) mathematically define the full 3-input XOR function. |

## UNIT-III: Combinational and Sequential Circuit Architecture

Unit III constitutes the vital bridge connecting abstract logic formulation to physical architectural components. Combinational circuits (such as adders, multiplexers, and decoders) evaluate instantaneous present-state logic, lacking memory. In stark contrast, sequential circuits (flip-flops, registers, and counters) introduce clock-driven memory states, allowing systems to track temporal progression.

Evaluative matrices in this domain frequently target structural cascading traps. For instance, calculating the requisite logic gates to cascade multiple multiplexers—determining that an  multiplexer constructed from  variants requires exactly  components (e.g., a  MUX requires fifteen  MUXes) is a staple of medium-to-hard tier questioning.19 In sequential logic, transition and excitation tables form the analytical core. Converting a JK flip-flop to a Toggle (T) flip-flop by merging specific inputs is a classic assessment paradigm designed to test hardware intuition.20 Furthermore, counters challenge candidates to differentiate between synchronous (parallel clock distribution) versus asynchronous (ripple clock propagation) delay physics.21

### Unit III Assessment Matrix

| # | Difficulty | Question | Options | Correct Answer | Detailed Pedagogical Explanation |
| --- | --- | --- | --- | --- | --- |
| 81 | Easy | A digital circuit designated to add two 1-bit operands to produce both a Sum and a Carry output is termed a: | A) Full Adder B) Half Adder C) Multiplexer D) Magnitude Comparator | B | A Half Adder is constrained to adding only two bits, lacking the capability to accept a Carry-in from a previous stage. |
| 82 | Easy | Which combinatorial circuit acts functionally as a "Data Selector"? | A) Decoder B) Demultiplexer C) Multiplexer D) Priority Encoder | C | A Multiplexer logically selects one of available data inputs to route to a single unified output line. |
| 83 | Easy | A line decoder features exactly how many primary input selection lines? | A) 1 B) 3 C) 8 D) 11 | B | It decodes a 3-bit binary input vector into 8 distinct, mutually exclusive active output lines. |
| 84 | Easy | Which sophisticated flip-flop architecture successfully eliminates the invalid/indeterminate state inherent in the basic SR flip-flop? | A) D Flip-flop B) T Flip-flop C) JK Flip-flop D) Master-Slave Latch | C | The JK flip-flop intelligently toggles its output state when both input lines are driven to logic 1, resolving the SR invalid state. |
| 85 | Easy | A combinational circuit's instantaneous output state depends strictly and entirely upon: | A) Historical past inputs B) Current present inputs C) The leading clock edge D) Internal memory latches | B | Combinational logic is devoid of memory elements; outputs rely purely on immediate input signal propagation. |
| 86 | Easy | Which hardware device is utilized to translate binary/BCD codes into corresponding Seven Segment display actuation signals? | A) Multiplexer B) Priority Encoder C) Decoder D) Ripple Counter | C | A specialized decoder translates the 4-bit BCD input into 7 distinct lines to illuminate the proper LED segments. |
| 87 | Easy | Structurally, sequential circuits consist of combinational logic tightly paired with what crucial component? | A) Memory elements B) Operational amplifiers C) Tri-state buffers D) The ALU | A | Memory elements (flip-flops or latches) capture and store the logical state across clock cycles. |
| 88 | Easy | Which specific register variant shifts logical data laterally, one bit position at a time? | A) Parallel In Parallel Out B) Shift Register C) Buffer Register D) Asynchronous Counter | B | Shift registers physically move data bits laterally through a chain of flip-flops upon sequential clock pulses. |
| 89 | Easy | An active-HIGH SR latch subjected to input conditions will immediately transition to which state? | A) Set B) Reset C) Invalid D) Hold | B | A high state on the Reset (R) terminal unequivocally forces the Q output to a logical 0. |
| 90 | Easy | Which sequential memory device natively toggles its output state upon every active clock edge when asserted? | A) T Flip-flop B) D Flip-flop C) SR Flip-flop D) Transparent Latch | A | The T (Toggle) flip-flop changes its internal state if, and only if, the input is held at logic 1 during the clock pulse. |
| 91 | Easy | A combinational circuit explicitly comparing two operands and to determine if , , or is known as a: | A) Parallel Adder B) Code Converter C) Magnitude Comparator D) Decoder array | C | A magnitude comparator evaluates the relative numerical magnitude of two binary inputs, activating corresponding state outputs. |
| 92 | Easy | What is the fundamental mathematical characteristic equation defining a D flip-flop? | A) B) C) D) | A | The next state directly mirrors the D (Data) input, ignoring the current state. |
| 93 | Easy | A complete Full Subtractor circuit physically incorporates how many input terminals? | A) 2 B) 3 C) 4 D) 1 | B | The circuit requires three inputs: the Minuend, the Subtrahend, and the Borrow-in from a previous stage. |
| 94 | Easy | Which class of flip-flop is primarily and almost exclusively utilized to construct shift registers? | A) SR B) T C) D D) JK | C | D flip-flops easily and predictably pass data sequentially from one stage to the next without toggling risks. |
| 95 | Easy | In an asynchronous (ripple) counter topology, the master clock signal is applied physically to: | A) All flip-flops simultaneously B) Only the first flip-flop C) Only the final flip-flop D) The clear terminals | B | Only the first stage receives the master clock; the output of each stage acts as the clock for the subsequent stage.21 |
| 96 | Easy | The theoretical minimum number of NAND gates required to fully synthesize a Half Adder circuit is: | A) 3 B) 4 C) 5 D) 6 | C | It demands exactly 5 interconnected NAND gates to simultaneously generate both the Sum (XOR equivalent) and Carry (AND equivalent) logic. |
| 97 | Medium | Exactly how many multiplexers are required to construct a hierarchical multiplexer? | A) 8 B) 15 C) 16 D) 31 | B | The hierarchical routing tree requires 8 MUXes in the first stage, then 4, then 2, then 1, totaling 15 multiplexers.19 |
| 98 | Medium | A Full Adder circuit can be structurally synthesized by combining: | A) Two Half Adders and an OR gate B) Two Half Adders and an AND gate C) One Half Adder and an XOR gate D) Three Half Adders | A | The two partial sums combine via a secondary XOR, while the partial carries combine via an OR gate to form the final Carry-out. |
| 99 | Medium | To implement an arbitrary 4-variable Boolean function using a Multiplexer, the minimum size MUX required is: | A) 4:1 B) 8:1 C) 16:1 D) 32:1 | B | An MUX processes 3 variables via its select lines; the 4th variable is evaluated and fed into the data input pins. |
| 100 | Medium | What is the total modulus (number of unique states) of a standard 4-bit ripple counter? | A) 4 B) 8 C) 15 D) 16 | D | A 4-bit counter physically sequences from binary 0000 to 1111, yielding exactly 16 distinct combinational states (). |
| 101 | Medium | Which binary code ensures that only a single bit changes between any two successive values, highly useful in minimizing errors in mechanical encoders? | A) BCD B) ASCII C) Gray Code D) Excess-3 | C | Gray code is a specialized unweighted code designed specifically for 1-bit unit distance transitions.21 |
| 102 | Medium | Within a standard JK flip-flop, the detrimental "race-around condition" manifests when: | A) B) C) D) and pulse width is excessive | D | Continuous rapid toggling during a long active clock pulse causes severe output instability before the clock edge falls.21 |
| 103 | Medium | A Master-Slave flip-flop dual-stage configuration was engineered specifically to solve what persistent problem? | A) Excessive power consumption B) Propagation delay C) The Race-around condition D) Indeterminate states | C | The slave stage edge-triggers only after the master stage locks, decisively preventing continuous feedback toggling. |
| 104 | Medium | To rapidly convert a JK flip-flop into a Toggle (T) flip-flop through hardwiring: | A) Connect J and K to ground B) Short J and K together C) Connect J to Q' D) Invert K | B | Shorting J and K creates a single T input node (), replicating toggle functionality.20 |
| 105 | Medium | A serial-in shift register requires exactly how many discrete clock pulses to fully load 4 bits of external data? | A) 1 B) 2 C) 3 D) 4 | D | Serial loading moves data linearly one bit at a time, requiring one full clock pulse per bit ingested. |
| 106 | Medium | The difference output of a full subtractor mathematically matches which specific output of a full adder? | A) Half Subtractor difference B) Full Adder Sum C) Comparator equality D) Decoder output | B | Both outputs are mathematically defined by the exact same Boolean XOR sequence: .21 |
| 107 | Medium | An -bit synchronous counter is driven by a master clock frequency . What is the observed frequency at the output of the final flip-flop stage? | A) B) C) D) | C | Each flip-flop stage physically divides the incoming clock frequency by 2, resulting in exponential reduction. |
| 108 | Medium | A combinational logic device capable of routing data from a single input line to one of multiple specific output lines is termed a: | A) Multiplexer B) Demultiplexer C) Encoder D) Parallel Adder | B | A Demultiplexer functions as an intelligent data distributor, reverse-routing compared to a MUX. |
| 109 | Medium | Synthesizing Excess-3 code from standard BCD code requires the logical addition of what specific binary value? | A) 0011 B) 0110 C) 1001 D) 0101 | A | Excess-3 is generated algorithmically by adding decimal 3 (binary 0011) to each discrete BCD digit. |
| 110 | Medium | To construct an abbreviated Mod-12 counter, the theoretical minimum number of flip-flops required is: | A) 3 B) 4 C) 5 D) 12 | B | Four flip-flops provide up to 16 native states (), which is the minimum power of two . |
| 111 | Medium | Which specific encoder variant prioritizes and encodes the highest-value active input line while ignoring lower-value inputs? | A) Binary Encoder B) Priority Encoder C) Octal Encoder D) Hexadecimal Encoder | B | A Priority encoder resolves simultaneous active inputs by outputting the binary code of the highest-ranked line. |
| 112 | Medium | The standard excitation table for a D flip-flop mathematically reveals that the required input is always identical to: | A) The Present state B) The Inverted state C) The Next state D) The Clock signal | C | To achieve any target state , the D input must be driven to exactly equal . |
| 113 | Hard | Analyzing a JK flip-flop excitation table, forcing a transition from Present State 0 to Next State 1 strictly mandates inputs J and K to be: | A) B) C) D) | B | Asserting guarantees the next state becomes 1; the state of K is entirely irrelevant (Don't Care). |
| 114 | Hard | A 4-bit synchronous UP counter sits at state 1011. What is the logic state of the flip-flop inputs assuming T-type flip-flops are utilized? (Format: T3 T2 T1 T0) | A) 0 1 0 1 B) 0 1 1 1 C) 1 0 1 1 D) 1 1 1 1 | A | T inputs evaluate the AND combination of previous Qs. T0=1, T1=Q0=1, T2=Q0Q1=0, T3=Q0Q1Q2=0. Thus, 0101 commands the correct bit toggles to reach 1100. |
| 115 | Hard | To optimally design a complex combinational circuit handling 5 inputs and 1 output using multiplexers, what is the most efficient single IC choice? | A) 16:1 MUX B) 32:1 MUX C) 8:1 MUX D) 64:1 MUX | A | A 16:1 MUX processes 4 select lines; the 5th variable (and its complement) is hardwired directly to the data inputs. |
| 116 | Hard | A standard ring counter physically initialized with the pattern 1000 exhibits exactly how many distinct valid states? | A) 4 B) 8 C) 15 D) 16 | A | A ring counter simply circulates the single '1' indefinitely, resulting in exactly distinct states for flip-flops. |
| 117 | Hard | A Johnson (Twisted Ring) counter constructed using a chain of 5 flip-flops generates a sequence of how many states? | A) 5 B) 10 C) 16 D) 32 | B | The twisted feedback loop of a Johnson counter yields exactly states, calculating to states. |
| 118 | Hard | The internal propagation delay of a single flip-flop is 20ns. What is the absolute maximum safe clock frequency for a 4-bit asynchronous (ripple) counter? | A) 50 MHz B) 25 MHz C) 12.5 MHz D) 100 MHz | C | Cumulative delay is . Safe operating frequency . |
| 119 | Hard | Which of the following describes the critical invalid state logic trap necessitating correction in parallel BCD adders? | A) Sum > 9 requires adding binary 0110 B) Sum > 15 requires adding 0110 C) Carry out acts as the sole validity indicator D) Subtraction mandates double 2's complement | A | Hardware BCD addition mathematically corrects sums exceeding the decimal bound of 9 by adding 6 (0110). |
| 120 | Hard | To physically realize the Boolean expression utilizing an MUX, which specific data input pins () must be tied to logic 1 (Vcc)? | A) B) C) D) | A | Extracting the minterms directly yields 1, 2, and 7. Consequently, the data input pins must be pulled HIGH. |

## UNIT-IV: Processor Organization, Instruction Formats, and Addressing Modes

Unit IV deconstructs the internal execution engine of the Central Processing Unit (CPU), emphasizing instruction flow, stack memory manipulations, and complex operand retrieval methodologies. The predominant cognitive assessment vectors target Addressing Modes and the mathematical constraints of Instruction Formats.

An established and notoriously difficult trap in objective exams concerns effective address (EA) calculations. In Program Counter (PC) relative addressing, candidates routinely overlook the temporal sequence: the hardware PC automatically increments to the next instruction's address before the relative offset is added to fetch the data.22 Similarly, Auto-increment versus Auto-decrement addressing modes pose chronological sequencing traps—distinguishing post-increment (use address then increment) from pre-decrement (decrement address then use).24 Instruction format calculations further challenge students by demanding precise logarithmic synthesis—calculating exactly how many bits remain for an immediate operand after deducting the opcode and multiple register identifiers.26

### Unit IV Assessment Matrix

| # | Difficulty | Question | Options | Correct Answer | Detailed Pedagogical Explanation |
| --- | --- | --- | --- | --- | --- |
| 121 | Easy | Which critical CPU register tracks execution flow by holding the memory address of the next instruction to be fetched? | A) Instruction Register (IR) B) Accumulator (AC) C) Program Counter (PC) D) Stack Pointer (SP) | C | The PC mathematically dictates the sequential and branching flow of all program execution.28 |
| 122 | Easy | The designated opcode field within an instruction explicitly specifies: | A) The physical operand address B) The operation to be performed C) The target PC value D) The processor interrupt status | B | Opcode is the binary instruction that tells the CPU decoder what specific hardware action (Add, Load, etc.) to trigger.26 |
| 123 | Easy | A strict stack-organized computer framework operates continuously on which data sequencing principle? | A) FIFO B) LIFO C) Round Robin D) Priority queueing | B | Stacks operate structurally on a Last-In-First-Out mechanism for all push and pop operations. |
| 124 | Easy | Which addressing mode structurally embeds the operand value directly within the bits of the instruction itself? | A) Immediate B) Direct C) Indirect D) Register | A | Immediate addressing requires zero additional memory fetches, as the data is the operand.5 |
| 125 | Easy | In theoretical Zero-address instruction formats, where are the operands assumed to be located? | A) Within CPU Registers B) The Accumulator C) The top of the Stack D) L1 Cache | C | Stack architecture inherently targets the top two elements of the stack for ALU operations, eliminating the need for explicit addresses. |
| 126 | Easy | Which discrete phase of the instruction cycle actually retrieves the data from memory based on the calculated Effective Address? | A) Fetch Instruction B) Decode Opcode C) Execute Instruction D) Write-back result | C | The Execute phase resolves the operand location, fetches the data, and performs the ALU operation. |
| 127 | Easy | The Instruction Register (IR) serves what explicit architectural purpose? | A) Pointing to memory addresses B) Holding the currently fetched instruction word C) Storing ALU results temporarily D) Managing the stack boundary | B | The IR physically retains the current instruction word so the control unit decode logic can constantly analyze it. |
| 128 | Easy | Which addressing mode explicitly specifies that the target operand is located physically inside a CPU register? | A) Direct memory B) Indirect C) Register D) Implied | C | Register addressing accesses data natively within the high-speed CPU registers, avoiding RAM latency. |
| 129 | Easy | A Reduced Instruction Set Computer (RISC) architecture is heavily characterized by: | A) Highly variable length instructions B) A massive array of complex addressing modes C) Reduced, simple, single-cycle instructions D) Direct memory-to-memory operations | C | RISC pipelines simplify opcodes to achieve rapid execution, usually completing one instruction per clock cycle.25 |
| 130 | Easy | Which fundamental data manipulation instruction logically flips all bits within a target register? | A) AND B) NEG C) NOT (Complement) D) XOR | C | The NOT (Complement) operation mathematically achieves the strict 1's complement of the data. |
| 131 | Easy | A standard PUSH hardware operation onto a memory stack executes in which sequence? | A) Increments SP, then stores data B) Decrements SP, then stores data C) Only stores data D) Only alters the SP | B | In standard top-down stack architecture, the Stack Pointer (SP) decreases to allocate space before the data is written. |
| 132 | Easy | Which addressing mode is naturally implied in the assembly instruction "ADD R1, R2"? | A) Register B) Immediate C) Direct D) Base | A | The instruction dictates that both operands reside securely within the internal CPU registers. |
| 133 | Easy | The conceptual disparity between high-level programming constructs and low-level machine execution capabilities is defined as the: | A) Execution gap B) Semantic gap C) Architectural gap D) Translation gap | B | Complex Instruction Set Computers (CISC) were historically engineered to close this semantic gap with dense, powerful instructions.25 |
| 134 | Easy | Which critical CPU component orchestrates all internal timing, gating, and synchronization? | A) The ALU B) The Cache Controller C) The Control Unit (CU) D) The System Buses | C | The CU acts as the central nervous system, directing data traffic and operational timing across the entire CPU. |
| 135 | Easy | What mode is actively utilized if the instruction's address field directly contains the physical memory address of the operand? | A) Indirect B) Direct C) Immediate D) Relative | B | Direct addressing points completely straight to the RAM location housing the data. |
| 136 | Easy | Which of the following operations is strictly classified as a Data Transfer operation rather than manipulation? | A) ADD B) SHIFT C) LOAD D) OR | C | LOAD physically moves data bits from memory into a CPU register without mathematically altering the bits. |
| 137 | Medium | In a custom instruction architecture defining exactly 45 distinct instructions, how many bits are strictly required for the opcode field? | A) 4 B) 5 C) 6 D) 7 | C | Using logarithms: bits, which supplies enough capacity to encode up to 64 distinct instructions.27 |
| 138 | Medium | If a processor utilizes a 32-bit instruction length, featuring a 6-bit opcode and two 5-bit register identifiers, what is the maximum bit-size of the remaining immediate operand field? | A) 14 bits B) 16 bits C) 18 bits D) 20 bits | B | Subtraction calculation: remaining bits.27 |
| 139 | Medium | Program Counter (PC) relative addressing determines the target Effective Address (EA) via which calculation? | A) EA = Address field directly B) EA = PC + Address field offset C) EA = Base Register + Address D) EA = Index Register + Address | B | A signed mathematical offset is dynamically added to the current value of the Program Counter.5 |
| 140 | Medium | Under Indirect Addressing mode, the address field embedded within the instruction logically refers to: | A) The actual data operand B) A pointer in memory housing the true address of the operand C) The address of the next instruction D) The top of the stack | B | The instruction provides an address pointing to a memory word that acts as a secondary pointer to the final data. |
| 141 | Medium | Which addressing mode was specifically engineered to optimize array processing and repetitive loop constructs? | A) Immediate B) Direct C) Indexed D) Implied | C | Indexed addressing effortlessly steps through sequential memory arrays by iteratively incrementing an index register. |
| 142 | Medium | The Auto-increment addressing mode structurally alters the index register's value at what specific point in time? | A) Before operand fetch B) After operand fetch C) Randomly during decode D) It does not alter it | B | The hardware retrieves the operand utilizing the current address, and subsequently post-increments the register for the next cycle.24 |
| 143 | Medium | Auto-decrement addressing mode is frequently leveraged by compilers to natively implement which data structure? | A) Queue B) Linked Array C) Stack D) Binary Tree | C | Pre-decrementing the address before data access accurately simulates a top-down stack PUSH operation. |
| 144 | Medium | Translating and evaluating the mathematical expression on a pure stack machine demands which sequence? | A) PUSH A, PUSH B, ADD, PUSH C, PUSH D, ADD, MUL B) ADD A B, ADD C D, MUL C) LOAD A, ADD B D D) No valid sequence | A | Zero-address stack architectures rigidly rely on Reverse Polish Notation (RPN) for expression evaluation. |
| 145 | Medium | A processor core houses 64 registers. A standard R-type instruction references 3 distinct registers. How many instruction bits are consumed entirely by register identifiers? | A) 6 B) 12 C) 18 D) 24 | C | 64 registers mathematically require 6 bits to identify uniquely. total bits.29 |
| 146 | Medium | Which dynamic addressing mode is structurally superior for relocating entire blocks of code seamlessly in memory during runtime? | A) Direct B) Indirect C) Base Register D) Immediate | C | Base register addressing allows massive program blocks to shift dynamically by merely updating the single base register pointer. |
| 147 | Medium | Within a strict one-address instruction format, the second target operand is implicitly assumed to be located in: | A) The Program Counter B) Main Memory C) The Accumulator D) The Instruction Register | C | The architecture dictates the Accumulator holds one operand implicitly and physically receives the resulting calculation. |
| 148 | Medium | What execution bottleneck does a 'Branch Delay Slot' in RISC architectures attempt to solve? | A) Cache miss latency B) Memory bandwidth bottlenecks C) Pipelining control hazards (branch penalties) D) ALU overflow handling | C | It mitigates the pipeline flush penalty by executing an independent instruction while the branch target address is resolving.25 |
| 149 | Medium | Which specific instruction alters the program execution sequence permanently and unconditionally? | A) JMP (Jump) B) ADD C) JZ (Jump if Zero) D) CMP | A | JMP aggressively forces the PC to latch a new address, disregarding all current ALU status flags. |
| 150 | Medium | A vintage 16-bit processor utilizes an instruction with a 4-bit opcode. How many individual memory locations can be directly addressed? | A) B) C) D) | A | The available address field is bits. The maximum addressable space is therefore or 4K locations. |
| 151 | Medium | What does strict "Orthogonality" in an instruction set design imply to a programmer? | A) Execution is always high speed B) All addressing modes can be applied symmetrically to all instructions C) Restricted register usage D) Short opcodes only | B | Orthogonality denotes a highly symmetrical, intuitive, and flexible architectural design lacking arbitrary restrictions. |
| 152 | Medium | A subroutine CALL instruction inherently performs which hardware operation behind the scenes? | A) Clears the stack completely B) Pushes the current PC to the stack C) Pops the PC from the stack D) Resets the PC to 0 | B | It fundamentally saves the critical return address onto the stack before violently branching to the subroutine. |
| 153 | Hard | A 2-word instruction is located at memory address 300. It utilizes PC-relative addressing with a declared offset of 50. What is the calculated Effective Address (EA)? | A) 350 B) 351 C) 352 D) 302 | C | Cognitive Trap: The PC naturally increments to 302 after fetching the complete 2-word instruction. The true EA is . |
| 154 | Hard | Consider a 32-bit CPU supporting 70 diverse instructions. An I-type format contains an Opcode, 2 Registers (6 bits each), and an Immediate. What is the maximum unsigned immediate value? | A) 8191 B) 16383 C) 4095 D) 32767 | A | Opcode = 7 bits. Registers = 12 bits. Immediate = bits. Max value is .26 |
| 155 | Hard | If a processor's Base Register contains 1000, its Index Register contains 20, and the instruction offset is 5, what is the EA in Base-Indexed with offset mode? | A) 1025 B) 1020 C) 1005 D) 25 | A | The hardware simply cascades the additions: EA = Base + Index + Offset = . |
| 156 | Hard | A processor utilizes expanding opcodes. If 4-bit opcodes allow for 15 two-address instructions, how many one-address instructions can be manufactured from the remaining 1 opcode (assuming 6-bit address fields)? | A) 16 B) 32 C) 64 D) 128 | C | The single "escape" opcode acts as a prefix, freeing up the 6 bits of the second address field, yielding exactly new instructions. |
| 157 | Hard | In a CPU where instruction execution is heavily interleaved through a pipeline, a Structural Hazard is triggered when: | A) Input data is not ready B) A branch is unexpectedly taken C) Two distinct instructions demand simultaneous use of the same physical hardware unit D) System interrupts fire | C | Structural hazards involve physical resource conflicts, such as two pipeline stages attempting to drive the memory bus simultaneously.30 |
| 158 | Hard | Why is Cache Addressing firmly NOT classified as a standard Instruction Addressing Mode? | A) It operates too fast for the CPU B) It is managed entirely transparently by the hardware, invisible to the programmer C) It requires microcode programming D) It uses strictly internal registers | B | Addressing modes define logical data access methodologies for programmers. Caching is a physical hardware optimization invisible to logical execution.5 |
| 159 | Hard | Which specific addressing mode is strictly utilized to retrieve functional pointers from memory before accessing the actual data? | A) Indexed Addressing B) PC-Relative Addressing C) Memory Indirect Addressing D) Base-Register Addressing | C | Memory indirect architecture first fetches a pointer from memory, then uses that pointer as an address to fetch the actual target data.24 |
| 160 | Hard | An array A is processed where each data element consumes 4 words. If an indexed mode is used to iterate, what numerical value serves as the multiplier for stride access? | A) 1 B) 2 C) 4 D) 5 | C | To access successive elements dynamically, the index counter must be mathematically multiplied by the element size (4 words) to calculate the correct stride offset. |

## UNIT-V: Input/Output Organization and Memory Hierarchy Stratification

Unit V encapsulates the periphery of the system and the vital stratification of memory technologies. The primary evaluation targets the severe velocity discrepancies between raw CPU computation and lethargic memory/IO access. The cognitive hurdles for candidates include calculating complex parameters for Direct Memory Access (DMA), contrasting hardware interrupt-driven mechanisms against inefficient polled routines, and unraveling the mathematics behind cache mapping.

In Cache Mapping, exam developers routinely probe the mathematical relationship between Main Memory capacity, Cache capacity, and block dimensions to derive Tag, Index, and Offset bit configurations.4 A classic numerical trap revolves around determining the hit ratio and computing the Effective Access Time (EAT), requiring precise arithmetic synthesis to account for miss penalties.31 Virtual memory mechanics, specifically tracking page fault algorithms (LRU, FIFO) and identifying Translation Lookaside Buffer (TLB) impacts, form the secondary advanced assessment tier.

### Unit V Assessment Matrix

| # | Difficulty | Question | Options | Correct Answer | Detailed Pedagogical Explanation |
| --- | --- | --- | --- | --- | --- |
| 161 | Easy | Which discrete level of memory functions directly as an ultra-high-speed buffer isolating the CPU from slower Main Memory? | A) Read-Only Memory (ROM) B) Flash Storage C) Cache Memory D) Secondary Magnetic Disk | C | Cache physically buffers frequently accessed instructions and data payloads to match the rapid execution speeds of the CPU core.33 |
| 162 | Easy | Which primitive I/O communication method mandates that the CPU constantly burn cycles checking the hardware device status? | A) Interrupt-driven I/O B) Direct Memory Access (DMA) C) Polling (Programmed I/O) D) Channel I/O | C | Polling is notoriously inefficient because it wastes vital CPU cycles actively querying unready devices in a tight loop.5 |
| 163 | Easy | The acronym DMA technically stands for: | A) Direct Memory Access B) Dynamic Memory Allocation C) Data Memory Array D) Digital Memory Address | A | DMA enables high-speed peripherals to bypass the CPU entirely, transferring data directly to or from the system RAM.34 |
| 164 | Easy | Which of the following specific memory architectures is inherently non-volatile? | A) SRAM B) DRAM C) Cache D) ROM | D | Read-Only Memory (ROM) is physically constructed to retain its data matrix indefinitely even when system power is completely severed.34 |
| 165 | Easy | Ascending the computer memory hierarchy pyramid, which storage tier represents the fastest access speeds and smallest capacities? | A) Level 2 Cache B) Main Memory (RAM) C) CPU Registers D) Magnetic Disk | C | CPU Registers are structurally etched directly into the processor core, operating synchronously at the exact frequency of the CPU.35 |
| 166 | Easy | What specific systemic event can trigger a hardware Interrupt? | A) External hardware signal B) Internal software trap/fault C) I/O device readiness notification D) All of the above | D | Interrupts are comprehensive asynchronous alerts generated by peripherals, catastrophic software faults (divide by zero), or external hardware traps. |
| 167 | Easy | Dynamic RAM (DRAM) is critically characterized by its strict architectural need to be: | A) Flashed via ultraviolet light B) Erased electrically C) Periodically refreshed D) Programmed only once | C | DRAM utilizes microscopic capacitors that physically leak electrical charge over milliseconds, mandating continuous refresh cycles to preserve data logic. |
| 168 | Easy | Which fundamental computing principle governs the statistical success of cache memory operation? | A) Principle of Locality of Reference B) Virtual memory mapping C) Sequential Polling D) Parallel Concurrency | A | Both spatial and temporal locality dictate statistically that data located near recently utilized data will be demanded again imminently. |
| 169 | Easy | The software-driven process of artificially expanding RAM capacity by utilizing segments of the hard disk is termed: | A) Cache layering B) Flash extension C) Virtual Memory D) ROM shadowing | C | Virtual memory dynamically swaps chunks of memory (pages) between physical RAM and disk, simulating an immense, unified memory space.36 |
| 170 | Easy | Which specific DMA transfer mode aggressively suspends the CPU for just a single clock cycle to transfer exactly one word of data? | A) Burst mode B) Block mode C) Cycle Stealing mode D) Demand transfer mode | C | Cycle stealing delicately takes control of the system bus for minimal, interspersed durations, strictly to prevent total CPU starvation. |
| 171 | Easy | Vectored interrupt architectures dictate that the interrupting hardware device must supply the CPU with: | A) The complete data payload B) The specific branch address/vector C) The updated stack pointer D) An execution opcode | B | The supplied vector points the CPU directly to the exact starting address of the corresponding Interrupt Service Routine (ISR) in memory.37 |
| 172 | Easy | A "Page Fault" exception occurs explicitly when: | A) The Cache completely misses B) The physical RAM is structurally full C) A logically requested page is not currently present in main memory D) The hard disk crashes mechanically | C | A page fault forces the Operating System to pause execution and fetch the missing data page from the slow secondary storage into RAM. |
| 173 | Easy | Isolated I/O (also known as I/O Mapped I/O) architecture ensures operations utilize: | A) Completely shared memory addresses B) A distinctly separate address space strictly for I/O C) Software Polling only D) Only internal registers | B | Distinct, segregated address spaces mathematically prevent peripheral I/O ports from consuming valuable RAM memory addresses. |
| 174 | Easy | Which specific classification of memory utilizes bistable flip-flop circuits to store data bits? | A) ROM B) DRAM C) SRAM D) EEPROM | C | Static RAM (SRAM) employs complex bistable latching circuitry (typically 6 transistors per cell), requiring absolutely no refresh cycles. |
| 175 | Easy | The absolute smallest addressable entity within computer memory organization is conventionally a: | A) Data Block B) Byte C) Cell D) Memory Page | C | Memory is structurally divided into microscopic cells, where each cell acts as a uniquely addressable location.38 |
| 176 | Easy | The systemic process of transporting blocks of data from a slower memory tier to a faster memory tier is formally termed: | A) Mapping B) Thrashing C) Fetching D) Swapping | A | Memory mapping mathematically transfers and translates blocks or pages up the hierarchy to optimize access speeds. |
| 177 | Medium | Assume Main memory size is and Cache size is . If the calculated index field is 12 bits, what is the precise size of the Tag field in Direct Mapping? | A) 7 bits B) 9 bits C) 12 bits D) 14 bits | B | Total address width = 21 bits (since ). tag bits.4 |
| 178 | Medium | In cache mapping mathematics, catastrophic "Thrashing" is most frequently observed in which mapping technique? | A) Direct Mapping B) Fully Associative C) Set-Associative D) Virtual Mapping | A | Constant swapping occurs because multiple heavily utilized memory blocks mathematically map to the exact same singular cache line.39 |
| 179 | Medium | Which sophisticated replacement algorithm systematically replaces the cache block that has not been referenced for the longest chronological time? | A) FIFO (First In First Out) B) LIFO (Last In First Out) C) LRU (Least Recently Used) D) Random Selection | C | LRU utilizes hardware counters to meticulously track the chronological usage history of blocks for optimal, logical replacement.30 |
| 180 | Medium | To successfully construct a unified memory module utilizing memory chips, exactly how many physical chips are required? | A) 2 B) 4 C) 8 D) 16 | B | Simple capacity division: Total capacity divided by individual chip capacity: chips.40 |
| 181 | Medium | What specialized hardware component massively accelerates Virtual Memory to physical memory address translation? | A) The DMA Controller B) The Translation Lookaside Buffer (TLB) C) A hardware Multiplexer D) The Interrupt Controller | B | The TLB acts physically as an ultra-fast associative cache dedicated entirely to storing recent page table translation entries. |
| 182 | Medium | An associative memory module (CAM) fundamentally works on the architectural principle of: | A) Address matching B) Data content matching C) Sequential block access D) LIFO processing | B | Content Addressable Memory (CAM) searches all cells concurrently by comparing input data against the data content, ignoring physical addresses entirely. |
| 183 | Medium | Under a strict "write-through" cache update policy: | A) Data is written exclusively to the L1 cache B) Data is written to memory only upon block replacement eviction C) Data is simultaneously updated in both the cache and the main memory D) Memory writes are queued indefinitely | C | This ensures extremely high data consistency across the hierarchy but incurs notoriously slower write performance times.10 |
| 184 | Medium | A Set-Associative cache architecture maps a given main memory block to: | A) Any random cache line B) One strictly specific cache line C) A specific set of lines, anywhere within that set D) Random sets | C | It bridges the gap between direct and fully associative by allowing a block flexibility to sit anywhere within a mathematically designated set.39 |
| 185 | Medium | The "Burst mode" DMA transfer protocol is most optimally suited for: | A) Keyboard stroke input B) Mouse movement tracking C) Magnetic disk sequential transfers D) Printer carriage control | C | High-speed, high-volume peripherals require locking the system bus to transfer massive, contiguous data blocks rapidly. |
| 186 | Medium | Implementing Memory-Mapped I/O implies structurally that: | A) I/O uses separate read/write command lines B) I/O devices physically share the identical CPU address space as RAM C) Main memory is stored within the I/O devices D) DMA hardware is disabled | B | No separate, specialized I/O instructions are needed; standard LOAD/STORE instructions transparently interact with the I/O ports.30 |
| 187 | Medium | Daisy Chaining is a physical hardware mechanism historically used for: | A) Cache block mapping B) Memory interleaving optimization C) Bus Arbitration and resolving Interrupt priority D) Connecting ROM chips | C | Hardware devices are connected linearly to dynamically grant acknowledgment priority to the device physically nearest to the CPU core. |
| 188 | Medium | A computer achieves a cache hit ratio of 80%. Cache access time is 40ns and Main Memory is 100ns. What is the calculated Effective Access Time (EAT)? | A) 40ns B) 52ns C) 60ns D) 140ns | C | Calculation: .31 |
| 189 | Medium | What does the "Valid Bit" integrated within a cache line metadata tag indicate? | A) The data has been modified (dirty) B) The data perfectly matches main memory C) The line contains legitimate, readable data D) The block is locked by the OS | C | It differentiates completely empty, stale, or powered-off cache lines from lines containing active, trustworthy data blocks. |
| 190 | Medium | To successfully interface a rapid processor with a highly slow, legacy memory module, the processor must be configured to: | A) Assert a hardware interrupt B) Insert cyclical WAIT states C) Deploy a DMA controller D) Stop the master clock entirely | B | Wait states intentionally freeze the processor's execution pipeline until the slow memory acknowledges that data is stable and ready. |
| 191 | Medium | The infamous "Cache Coherence" problem arises primarily in which architectural configurations? | A) Simple Uniprocessors B) Symmetric Multiprocessor systems (SMP) C) Virtual memory paging systems D) Solid State Flash drives | B | Multiple CPUs with isolated, private L1 caches modifying shared variables lead quickly to mathematically inconsistent data states across the system.4 |
| 192 | Medium | Which architectural implementation factor most effectively reduces the "Miss Penalty" in cache systems? | A) Expanding the L1 cache size B) Increasing set associativity C) Deployment of an L2 Cache layer D) Utilizing random replacement policies | C | A multi-level cache hierarchy (L2) prevents the massive penalty of fetching directly from highly latent RAM upon an L1 miss. |
| 193 | Hard | Assuming a main memory block is mathematically identified as . In a direct-mapped cache containing exactly lines, which exact line does block map to? | A) B) C) D) | C | The direct mapping algorithm is determined strictly by standard modulo arithmetic: Block Number modulo Total Cache Lines.32 |
| 194 | Hard | A memory system utilizes 24-bit addresses. The Block size is 4 words. A direct-mapped cache has 512 lines. What is the exact bit-size of the tag field? | A) 11 bits B) 13 bits C) 15 bits D) 9 bits | B | Words/block = 4 (, offset=2). Lines = 512 (, index=9). Tag = Total Address (24) - Index (9) - Offset (2) = 13 bits.41 |
| 195 | Hard | Consider an array A mapped to an initially empty cache featuring 16-word blocks. A loop executes causing an inevitable miss on . Assuming sequential access, what is the resultant statistical miss ratio? | A) 0.50 B) 0.25 C) 0.0625 D) 0.125 | C | 1 miss fetches an entire block of 16 words. Thus, you suffer 1 miss per 16 accesses .32 |
| 196 | Hard | In a 4-way set associative cache containing 16 blocks total, Main memory block 216 maps directly to which cache set? | A) Set 0 B) Set 1 C) Set 2 D) Set 3 | A | Total sets = Cache Blocks / Ways = Sets. Mapping equation = Block Number mod Total Sets = .32 |
| 197 | Hard | A page table resides deep in main memory (100ns access). The TLB hit ratio is a robust 90% with a 10ns access time. What is the Effective Access Time with the TLB active? | A) 110ns B) 120ns C) 130ns D) 140ns | B | EAT = Hit Rate * (TLB time + Mem time) + Miss Rate * (TLB time + Mem time for Page Table + Mem time for Data). . |
| 198 | Hard | The CPU is busy evaluating logic 80% of the time. A DMA controller transfers a block taking . If Cycle Stealing mode is deployed over Burst mode, what happens to the CPU's overall efficiency? | A) It plummets B) It increases slightly C) It remains completely unaffected D) It reaches a theoretical 100% | B | Cycle stealing interleaves memory transfers selectively, preventing the CPU from freezing entirely during the 1ms block transfer, optimizing throughput. |
| 199 | Hard | Cache Size is , Block Size is 32 Bytes. Main Memory is . Under a Fully Associative mapping scheme, exactly how many bits represent the Index field? | A) 0 B) 5 C) 10 D) 15 | A | Cognitive trap: A Fully associative mapping architecture utilizes absolutely no index bits; the address splits simply into the Tag and the Offset.39 |
| 200 | Hard | If physical RAM size is physically increased by adding modules to the motherboard, the size of the logical virtual memory: | A) Increases proportionally B) Decreases C) Remains completely unchanged D) Doubles | C | Virtual memory is bounded entirely by the processor's logical address architecture (e.g., 32-bit or 64-bit), remaining independent of physical RAM constraints. |

## Concluding Assessment Methodology and Exam Validation

The compilation of this exhaustive 200-question repository guarantees the multi-tiered cognitive evaluation required for mastering Computer Organization and Architecture. The rigorous integration of structural algorithms (such as Booth's signed methodology), logical reduction heuristics (encompassing 5 and 6-variable Karnaugh maps), and hierarchical memory mathematics effectively benchmarks student cognitive retention.

By distributing questions across highly precise difficulty spectrums, examiners can accurately profile both the superficial factual recall and the profound architectural synthesis of the examinee. The deliberate inclusion of cognitive traps—such as offset timing errors in PC-relative addressing, and index elimination in fully associative caches—guarantees that the exam discriminates effectively between students who have rote-memorized the syllabus and those who have internalized the spatial and temporal mechanics of computer architecture.

#### Works cited

- BCA Syllabus - 2025 - CHHATRAPATI SHAHU JI MAHARAJ UNIVERSITY, KANPUR Session 2025-26 onwards, accessed on April 9, 2026, https://csjmu.ac.in/wp-content/uploads/syllabus-c/02%202025%20Onwards/BCA%20Syllabus%20-%202025%20onwards.pdf

- Proposed Syllabus by C.S.J.M.University,Kanpur. Bachelors of Computer Application Semester, accessed on April 9, 2026, https://csjmu.ac.in/wp-content/uploads/pdf/BCA.pdf

- 5 variable K-Map in Digital Logic - GeeksforGeeks, accessed on April 9, 2026, https://www.geeksforgeeks.org/digital-logic/5-variable-k-map-in-digital-logic/

- Objective Question with Answer for Cache Mapping - Download Free PDF - Testbook, accessed on April 9, 2026, https://testbook.com/objective-questions/mcq-on-cache-mapping--5eea6a0a39140f30f369db0d

- Addressing Modes MCQ [Free PDF] - Objective Question Answer for Addressing Modes Quiz - Download Now! - Testbook, accessed on April 9, 2026, https://testbook.com/objective-questions/mcq-on-addressing-modes--5eea6a0a39140f30f369dacd

- Booth's Algorithm - GeeksforGeeks, accessed on April 9, 2026, https://www.geeksforgeeks.org/computer-organization-architecture/computer-organization-booths-algorithm/

- Explain the Booth's algorithm in depth with the help off low chart. Give an example for multiplication using Booth's algorithm. OR Discuss the Booth's algorithm for 2's complement number. Also illustrate it with the some example.OR Explain Booth's multiplication algorithm in detail. - EasyExamNotes.com, accessed on April 9, 2026, https://easyexamnotes.com/explain-the-booths-algorithm-in-depth-with-the-help-off-low-chart-give-an-example-for-multiplication-using-booths-algorithm-or-discuss-the-booths-algorithm-for-2s/

- Booth's Algorithm and Assembly Quiz | PDF | Computer Data Storage | Digital Electronics, accessed on April 9, 2026, https://www.scribd.com/document/735680084/quiz-answar

- Booth's Algorithm Exercise Uses Registers as follows • M, accessed on April 9, 2026, https://www.ecs.csun.edu/~cov/comp222s16/notes/exercise_booth.pdf

- MULTIPLE CHOICE QUESTIONS - COA, accessed on April 9, 2026, http://ggn.dronacharya.info/ECEDept/Downloads/QuestionBank/Vsem/QuizCOA.pdf

- Computer Architecture Multiple-Choice Questions - BYJU'S, accessed on April 9, 2026, https://byjus.com/gate/computer-architecture-mcqs/

- Larger 5 & 6-variable Karnaugh Maps - All About Circuits, accessed on April 9, 2026, https://www.allaboutcircuits.com/textbook/digital/chpt-8/larger-5-6-variable-karnaugh-maps/

- 6 Variable K-Map in Digital Electronics - TutorialsPoint, accessed on April 9, 2026, https://www.tutorialspoint.com/digital-electronics/six-variable-k-map-in-digital-electronics.htm

- Lecture 8 The Karnaugh Map or K-map, accessed on April 9, 2026, https://ggn.dronacharya.info/EEEDept/Downloads/QuestionBank/IVSem/DE/Section-A/8_K_map.pdf

- Karnaugh Maps (K-Map) | 1-6 Variables Simplification & Examples - Electrical Technology, accessed on April 9, 2026, https://www.electricaltechnology.org/2018/05/karnaugh-map-k-map.html

- Karnaugh Map Questions and Answers PDF - Gate Vidyalay, accessed on April 9, 2026, https://www.gatevidyalay.com/tag/karnaugh-map-questions-and-answers-pdf/

- 6 Variable K Map: Example and Boolean Function Optimization - YouTube, accessed on April 9, 2026, https://www.youtube.com/watch?v=vbJUQ4RaIog

- K-map Simplification MCQs | PDF | Chess Openings | Logic - Scribd, accessed on April 9, 2026, https://www.scribd.com/document/886583458/MCQs-Kmap

- Multiplexer MCQ [Free PDF] - Objective Question Answer for Multiplexer Quiz - Download Now! - Testbook, accessed on April 9, 2026, https://testbook.com/objective-questions/mcq-on-multiplexer--5eea6a1439140f30f369f282

- Objective Question with Answer for Sequential Circuits - Download Free PDF - Testbook, accessed on April 9, 2026, https://testbook.com/objective-questions/mcq-on-sequential-circuits--5eea6a0d39140f30f369e207

- Question Bank for Multiple Choice Questions Program: Diploma in Computer engineering Program Code:- CO Scheme:-I Semester:- 3 Co - Zeal Polytechnic Pune, accessed on April 9, 2026, https://zealpolytechnic.com/wp-content/uploads/2023/01/SY-COMP-Sem-III-DTE-22320-MCQs-Bank-1.pdf

- Objective Question with Answer for Machine Instructions and Addressing Modes - Download Free PDF - Testbook, accessed on April 9, 2026, https://testbook.com/objective-questions/mcq-on-machine-instructions-and-addressing-modes--5eea6a0a39140f30f369daf1

- All questions of Machine Instructions & Addressing Modes for Computer Science Engineering (CSE) Exam - EduRev, accessed on April 9, 2026, https://edurev.in/chapter/questions/8933/23146/Machine-Instructions-Addressing-Modes

- Computer Organization & Architecture MCQs: Addressing Modes & More - Studocu, accessed on April 9, 2026, https://www.studocu.vn/vn/document/truong-dai-hoc-fpt/computer-architecture/this-set-of-computer-organization-and-architecture-multiple-choice-questions/60903869

- Computer Organization MCQs Set-2 | PDF - Scribd, accessed on April 9, 2026, https://www.scribd.com/document/489001797/computer-organization-questions-and-answers-set-2-1

- Instruction Format MCQ [Free PDF] - Objective Question Answer for Instruction Format Quiz - Download Now! - Testbook, accessed on April 9, 2026, https://testbook.com/objective-questions/mcq-on-instruction-format--5eea6a0a39140f30f369daf3

- Computer Organization | Problem Solving on Instruction Format - GeeksforGeeks, accessed on April 9, 2026, https://www.geeksforgeeks.org/computer-organization-architecture/computer-organization-problem-solving-instruction-format/

- Top 55 Computer Organization and Architecture MCQs - NxtWave, accessed on April 9, 2026, https://www.ccbp.in/blog/articles/computer-organization-and-architecture-mcq

- Problem Solving on Instruction Format - Computer Architecture and Organisation - EduRev, accessed on April 9, 2026, https://edurev.in/t/187292/Problem-Solving-on-Instruction-Format

- Computer Architecture MCQ, accessed on April 9, 2026, https://www.bcetodisha.ac.in/elearning/content/cse_uploads/4th/MCQ%20COA.pdf

- Direct Mapped Cache Design - Virtual Lab for Computer Organisation and Architecture - IIT Kharagpur, accessed on April 9, 2026, https://cse.iitkgp.ac.in/~chitta/coldvl/dmc.html

- Quiz about Cache and main memory - GeeksforGeeks, accessed on April 9, 2026, https://www.geeksforgeeks.org/quizzes/cache-and-main-memory/

- Memory Hierarchy MCQ [Free PDF] - Objective Question Answer for Memory Hierarchy Quiz - Download Now! - Testbook, accessed on April 9, 2026, https://testbook.com/objective-questions/mcq-on-memory-hierarchy--5eea6a0a39140f30f369daff

- Objective Question with Answer for Computer Organization and Architecture - Download Free PDF - Testbook, accessed on April 9, 2026, https://testbook.com/objective-questions/mcq-on-computer-organization-and-architecture--5eea6a0a39140f30f369dacc

- Computer Memory Organization MCQs 2024 | PDF - Scribd, accessed on April 9, 2026, https://www.scribd.com/document/512601074/COA-mcq-1

- 30 MCQ on Computer Memory, accessed on April 9, 2026, https://www.cbmceindia.com/blog/details/30-mcq-on-computer-memory/

- Computer Organization and Architecture Multiple Choice Questions :- - Raghunathpur College, accessed on April 9, 2026, https://elearning.raghunathpurcollege.ac.in/files/1518D7D315884386530.pdf

- Top 62 Computer Memory MCQ Questions - Scientech Easy, accessed on April 9, 2026, https://www.scientecheasy.com/2021/08/computer-memory-mcq.html/

- Cache Memory MCQs with Answers | PDF - Scribd, accessed on April 9, 2026, https://www.scribd.com/document/970925697/Lec8-COA-Sheet

- Computer Organization Question Bank | PDF | Computer Data ..., accessed on April 9, 2026, https://www.scribd.com/document/802826221/Question-Bank-Computer-Organization-BCS

- CSCI 4717 – Memory Hierarchy and Cache Quiz, accessed on April 9, 2026, https://faculty.etsu.edu/tarnoff/tsts4717/Memory_Hierarchy_and_Cache_Quiz_Answers.pdf